Write a program for error detecting code using CRC CCITT (16-bits).

#include <stdio.h>

#include <stdint.h>

#include <string.h>

#define GEN_POLY 0x1021 // CRC-CCITT polynomial

#define GEN_POLY_BIT_SIZE 16

// Function to calculate CRC-CCITT checksum

uint16_t calculate_crc(const char *message, int message_length) {

uint16_t crc = 0xFFFF; // Initialize CRC to 0xFFFF

for (int i = 0; i < message_length; i++) {

crc ^= (message[i] << (GEN_POLY_BIT_SIZE - 8)); // XOR with the next byte

for (int j = 0; j < 8; j++) {

if (crc & 0x8000) {

crc = (crc << 1) ^ GEN_POLY;

} else {

crc = crc << 1;

}

}

}

return crc;

}

int main() {

char message[128];

printf("Enter data: ");

scanf(" %[^'\n']s", message); // Read the entire input line including spaces

int message_length = strlen(message);

char modified_message[128];

strcpy(modified_message, message);

// Calculate CRC-CCITT checksum

uint16_t checksum = calculate_crc(modified_message, message_length);

printf("\nOriginal Message: %s\n", message);

printf("CRC-CCITT Checksum: 0x%04X\n", checksum);

// Test error detection

int error_choice;

printf("Test error detection (0 for yes, 1 for no): ");

scanf("%d", &error_choice);

if (error_choice == 0) {

int error_position;

printf("Enter the position where error is to be inserted (1-%d): ", message_length);

scanf("%d", &error_position);

if (error_position >= 1 && error_position <= message_length) {

modified_message[error_position - 1] = (modified_message[error_position - 1] == '0') ? '1' : '0';

printf("Modified Message: %s\n", modified_message);

}

}

// Recalculate CRC-CCITT checksum

uint16_t new_checksum = calculate_crc(modified_message, message_length);

if (new_checksum == 0) {

printf("No error detected.\n");

} else {

printf("Error detected.\n");

}

return 0;

}

Write a program for congestion control using Leaky bucket algorithm.

#include <stdio.h>

#include <stdlib.h>

#include <unistd.h>

#include <time.h>

#define BUCKET_CAPACITY 10 // Bucket capacity in tokens

#define OUTPUT_RATE 2      // Output rate in tokens per second

// Function to perform traffic shaping using the Leaky Bucket algorithm

void leaky_bucket(int input_rate) {

int bucket = 0; // Token bucket initially empty

int time = 0;   // Simulation time

while (1) {

printf("\nTime: %d seconds - Bucket: %d tokens\n", time, bucket);

// Generate incoming packets

int incoming_packets = rand() % (input_rate + 1);

// Add incoming tokens to the bucket

bucket += incoming_packets;

// Limit the bucket capacity

if (bucket > BUCKET_CAPACITY) {

bucket = BUCKET_CAPACITY;

}

// Transmit data at the output rate

int transmitted = (bucket > OUTPUT_RATE) ? OUTPUT_RATE : bucket;

bucket -= transmitted;

// Increment simulation time

time++;

// Sleep for 1 second to simulate real-time behavior

sleep(1);

// Check for termination

if (time >= 30) {

break;

}

}

}

int main() {

int input_rate;

printf("Enter input rate (packets per second): ");

scanf("%d", &input_rate);

srand(time(NULL)); // Seed the random number generator

printf("\nStarting Leaky Bucket Congestion Control Simulation\n");

leaky_bucket(input_rate);

printf("\nSimulation complete.\n");

return 0;

}

Using TCP/IP sockets, write a client-server program to make client sending the file name and the server to send back the contents of the requested file if present.

serverTCP.py

from socket import *

serverName = "127.0.0.1"

serverPort = 12000

serverSocket = socket(AF_INET, SOCK_STREAM)

serverSocket.bind((serverName, serverPort))

serverSocket.listen(1)

while 1:

print("The server is ready to receive")

connectionSocket, addr = serverSocket.accept()

sentence = connectionSocket.recv(1024).decode()

file = open(sentence, "r")

l = file.read(1024)

connectionSocket.send(l.encode())

print('\nSent contents of ' + sentence)

file.close()

connectionSocket.close()

clientTCP.py

from socket import *

serverName = '127.0.0.1'

serverPort = 12000

clientSocket = socket(AF_INET, SOCK_STREAM)

clientSocket.connect((serverName, serverPort))

sentence = input("\nEnter file name: ")

clientSocket.send(sentence.encode())

filecontents = clientSocket.recv(1024).decode()

print('\nFrom Server:\n')

print(filecontents)

clientSocket.close()

Using UDP sockets, write a client-server program to make client sending the file name and the server to send back the contents of the requested file if present.

serverUDP.py

from socket import *

serverPort = 12000

serverSocket = socket(AF_INET, SOCK_DGRAM)

serverSocket.bind(("127.0.0.1", serverPort))

print("The server is ready to receive")

while 1:

sentence, clientAddress = serverSocket.recvfrom(2048)

sentence = sentence.decode("utf-8")

file = open(sentence, "r")

con = file.read(2048)

serverSocket.sendto(bytes(con, "utf-8"), clientAddress)

print('\nSent contents of ', end=' ')

print(sentence)

#  for i in sentence:

#  print (str(i), end = '')

file.close()

clientUDP.py

from socket import *

serverName = "127.0.0.1"

serverPort = 12000

clientSocket = socket(AF_INET, SOCK_DGRAM)

sentence = input("\nEnter file name:  ")

clientSocket.sendto(bytes(sentence, "utf-8"), (serverName, serverPort))

filecontents, serverAddress = clientSocket.recvfrom(2048)

print('\nReply from Server:\n')

print(filecontents.decode("utf-8"))

# for i in filecontents:

# print(str(i), end = '')

clientSocket.close()

clientSocket.close()